# flexible composite rail structural snow fence Specifications

## Design standard

- Meet or exceed 25 pounds per ft² (100 mph) wind load

- 30 year or greater life in the field

- Available in incremental fence design heights of 4’, 6’, 8’, and 10’

- Continuous fence lengths (up to 2 miles in length) to avoid fence breaks or offsets

- No external guy wires.  Utilize all internal bracing for interior support

- Adjustable fence porosities of 50% to 25%

- Minimum 6’ frost depth for the foundation design

- On Federally Funded Projects Meet Buy America Requirements 635.410 Buy America

- Screws, nuts, washers, and bolts shall be galvanized in accordance with Section 3392 and footing sleeves in accordance with Sections 3392 and 3394 of MnDOT’s Standard Specifications for Construction

- Electrical grounding per Section 2557 MnDOT Standard Specifications for Construction

- Above ground metal components powder coated Sierra Brown color or similar as approved by MnDOT

- Oscillation dampers shall be poly coated wire and Sierra Brown color or similar as approved by MnDOT

- Ability to adjust bottom fence gap (4 inches to 12 inches) from bottom of fence rail to top of ground to meet the fence design height bottom gap requirement

- Ability to install fence rails in an alternating woven pattern around posts

### Fence rails

- Fence rails shall be Sierra Brown color or similar as approved by MnDOT

- Rail height 6 inches (tolerance of 5.875 to 6.125 inches)

- Rail thickness (tolerance of .050 to .075 inches)

- Ultra Violet light stabilized Polyethylene Plastic

- May contain up to 20% Polyethylene Plastic pre-consumer recycled content

- 4 Strands of Galvanized Aircraft Cable, ASTM Class 3, with minimum breaking strength 900 pounds per cable or greater molecularly bonded to plastic

- Shall be secured to the line posts in a flexible manner to achieve the desired fence porosity and bottom fence gap

- Provide tensioning hardware to maintain a taut rail and minimize rail oscillation

### sleeves-FOR BOTH END AND lINE pOSTS

- Made of Steel- ASTM A500-18 Grade B&C

- Sleeve lengths of 6’, 8’, 10’, and 12’

- Maximum out of ground stub height of 3”

- Minimum embedment depth of 60”

- Must provide shear bolts for any fasteners requiring a set torque

- End post sleeves and any posts with tensioning structures need to be designed to be anchored in concrete

- All nested posts must be the same gauge to assure proper fit

| Milling Specifications for Footing Sleeves |  |  |  |
| --- | --- | --- | --- |
| Material |  |  | Certification |
| Used For | Type | Size (Inch) |  |
| Line Post Footing Sleeve | Tube | 4 x 3 x .188 | ASTM A500-18 GRADE B&C |
| End Post Footing Sleeve | Tube | 5 x 3 x .188 | ASTM A500-18 GRADE B&C |
| End Post Footing Sleeve | Tube | 5 x 4 x .188 | ASTM A500-18 GRADE B&C |

END POST SLEEVES

- Will not have soil stabilizer fins

LINE POST SLEEVES

- Shall have 4 soil stabilizer fins permanently welded

- Each stabilizer fin is 0.125 inches X 2 inches X 13 inches

- Each stabilizer fin is attached 3 inches below top of footing sleeve

### FENCE POSTS

- Made of Steel-ASTM A500-18 Grade B&C, or ASTM A992-11, or ASTM A572-15

- Either I-Beams or Hollow Structural Sections are acceptable provided they meet the wind load

- Provide rail attachment locations to achieve desired fence porosity

| Milling Specifications for Posts |  |  |  |
| --- | --- | --- | --- |
| Material |  |  | Certification |
| Used For | Type | Size (Inch) |  |
| Line Post | Tube | 3 x 2 x .25 | ASTM A500-18 GRADE B&C |
| End Post | Tube | 4 x 3 x .375 | ASTM A500-18 GRADE B&C |
| Line Post | Standard I-Beam | S3 x 5.7# | ASTM A992-11 |
| End Post | Standard I-Beam | S4 x 7.7# | ASTM A572-15 |

### hardware/materials

- Furnish brackets and hardware to properly attach the fence rails to the fence posts

- Provide winches for adjusting rail tension

| Milling Specifications for Flat work |  |  |  |
| --- | --- | --- | --- |
| Material |  |  | Certification |
| Used For | Type | Size (Inch) |  |
| Flat Steel | Flat | 1/4 " or less | ASTM A36C |
| Flat Steel | Flat | greater than 1/4" | ASTM A572 |

- Adjustable horizontal/angle bracing to keep the fence posts straight and avoid the use of guy wires

| Milling Specifications for Bracing |  |  |  |
| --- | --- | --- | --- |
| Material |  |  | Certification |
| Used For | Type | Size (Inch) |  |
| Adjustable Turnbuckle | Round | 2 x 2 x 1.24 | ASTM A500-18 GRADE B&C |
| Threads of Turnbuckle | Rod |  | A307 Grade A or B |

- Provide poly coated wire oscillation dampers Sierra Brown in color or similar as approved by MnDOT to minimize fence rail oscillation

- Provide all the necessary hardware, including electrical grounds, for putting the fence together

- Concrete for footings furnished by others

### Technical support

- Provide and coordinate distribution of Pounder Plate for driving the line post sleeves

- Provide and coordinate distribution of Rail Payout Device for unspooling the rail during fence installation

- Provide on-site representation when called upon, as well as, being available via phone or e-mail in a timely manner for assistance during project design, construction, and maintenance phases